Transforming ENU Displacement Time Series

In practice, users typically work with local ENU displacement time series derived from IGS20 solutions. These can be transformed into the GOM25 frame by applying the one-set 7-parameter transformation to their ECEF-XYZ coordinates as an intermediate step. The procedure is:

Step 1. Estimate ECEF-XYZ at the site

A geographical location of the site defined by latitude (ϕ) and longitude (λ) can be estimated from the site log or other sources. Small datum differences (e.g., WGS84 vs. IGS20) or ellipsoidal height errors of a few meters introduce negligible changes in the local ENU rotation and the resulting transformed displacements or velocities. These errors are typically overshadowed by other noise sources in multi-year geodetic time series. For most practical applications, approximate station latitude and longitude (e.g., from Google Earth) are sufficient to transform ENU displacement time series or site velocities between IGS20 and GOM25.

From the geodetic ϕ and λ (radians) and an approximate ellipsoidal height (h), the ECEF-XYZ coordinates P0=(X0, Y0, Z0) in the IGS20 reference frame can be calculated according to the following equations (Eq. 1). If the ellipsoidal height is negligible or unknown, it can be assumed zero (or an approximate value).

X0=(N+h) cosϕ cosλ

Y0=(N+h) cosϕ sinλ                     (1)

Z0=(N1-e2+h)sinϕ

where N is the prime-vertical radius of curvature, also known as radius of curvature in the prime vertical, or normal radius of curvature:

N=a1-e2sin2ϕ                (2)

Where, a=6378137.0 m (GRS80 semi-major axis), f=1/298.257222101 (flattening), and e2=2f-f2 .

Step 2. Convert ENU(IGS20) Displacements to XYZ(IGS20)

Convert each epoch’s ENU displacements to XYZ displacements using the local rotation.

ΔrXYZIGS20=RENU2XYZ×ENU           (3)

ΔrXYZIGS20 represent a 3D displacement vector in the ECEF-XYZ coordinate system, i.e., its components are (ΔX, ΔY, ΔZ). RENU2XYZ represents a 3×3 rotation matrix that transforms a local ENU vector (E, N, U) into the XYZ coordinate system at the station’s reference point (X0, Y0, Z0):

RENU2XYZ=-sinλ-sinλcosλ​cosϕcosλcosλ-sinϕsinλcosϕsinλ0cosϕsinϕ.                           (4)

Then form absolute ECEF-XYZ coordinates at epoch t:

PIGS20t=P0+ΔrXYZIGS20(t)…………(5)

Step 3. Transform XYZ(IGS20) to XYZ(GOM25)

The ECEF-XYZ(IGS20) coordinates can be transformed to GOM25 applying the one-set 7-parameters.

Step 4. Convert XYZ(GOM25) to ENU(GOM25)

Subtract the original reference point (X0, Y0, Z0) to get the displacements in XYZ coordinates:

ΔrXYZGOM25=(XGOM25-X0, YGOM25-Y0, ZGOM25-Z0)                   (6)

Rotate these displacements back into the ENU frame:

EGOM25NGOM25UGOM25=RENU2XYZT×ΔrXYZGOM25                      (7)

This yields the local ENU displacement time series with respect to GOM25.

The reverse transformation of ENU displacement time series from GOM25 to IGS20 follows the same logic but applies the inverse XYZ transform in Step 3, followed by the same ENU/XYZ rotations.

Transforming ENU Velocities

This section describes the method for transforming site velocities between the IGS20 and GOM25 reference frames when velocities are provided in East-North-Up directions. The transformation is derived by differentiating the one-set 7-parameter transformation model, providing a direct and efficient conversion for practical applications.

Step 1: Estimate position at the alignment epoch.
The initial ECEF-XYZ position at the site P0=(X0, Y0, Z0) can be estimated according to Eq. 1.

Step 2: Convert ENU-Velocities (IGS20) to XYZ-Velocities (IGS20)

This can be done by

VXYZIGS20=RENU2XYZ×VENUIGS20                          (8)

RENU2XYZ is defined in Eq. 4.

Step 3. Convert VXYZIGS20 to VXYZGOM25

The XYZ velocities with respect to GOM25 can be obtained by differentiating the one-set 7-parameter position transform:

VXYZGOM25=VXYZIGS20+T'-R'×P0.                                (9)

Step 4. Convert VXYZGOM25 to VENUGOM25

Finally, convert the ECEF velocities to ENU velocities:

VENUGOM25=RENU2XYZT×VXYZGOM25                              (10)

Reverse conversion from GOM25 to IGS20 of ENU site velocities follows the same steps, but using the reverse XYZ-velocity transform in Step 3:

VXYZIGS20=VXYZGOM25-T'+R'×P0                               (11)

To facilitate practical adoption of GOM25, we provide a suite of open-source Python tools implementing the transformations described above, permanently archived with documentation (Wang and Welch 2025).